## Safe Work Practice

### Drills

How do you select the proper bit or attachment?

  - Follow manufacturers' instructions when selecting and using a bit or attachment, especially with unfamiliar drills or work.

  - Select the bit or attachment suitable for the size of the drill and the work being done.

  - Ensure that the bit or attachments are properly seated and tightened in the chuck.

  - Use only bits and attachments that turn true.

  - Use the auxiliary (second) handle for larger work or continuous operation.

### What should you do when working with powered hand drills?

  - Wear safety glasses or a face shield (with safety glasses or goggles).

  - Keep drill air vents clear to maintain adequate ventilation.

  - Keep drill bits sharp always.

  - Keep all cords clear of the cutting area during use. Inspect for frays or damage before each use.

  - Disconnect power supply before changing or adjusting bit or attachments.

  - Tighten the chuck securely. Remove chuck key before starting drill.

  - Secure work piece being drilled to prevent movement.

  - Slow the rate of feed just before breaking through the surface.

  - Drill a small "pilot" hole before drilling large holes.

### What should you do when working with small pieces?

  - Clamp stock so work will not twist or spin.

  - Do not drill with one hand while holding the material with the other.

### What should you avoid when working with powered hand drills?

  - Do not use a bent drill bit.

  - Do not exceed the manufacturer's recommended maximum drilling capacities.

  - Do not use a hole saw cutter without the pilot drill.

  - Do not use high speed steel (HSS) bits without cooling or using lubrication.

  - Do not attempt to free a jammed bit by starting and stopping the drill. Unplug the drill and then remove the bit from the work piece.

  - Do not reach under or around stock being drilled.

  - Do not overreach. Always keep proper footing and balance.

  - Do not raise or lower the drill by its power cord.

  - Do not use in wet or muddy locations. Use a nonpowered drill instead.

  - Do not use excessive force to drill into hard material. Reduce drill speed if possible.